Course Title : Computer Network

Course Code : CCE - 313

Final Assignment in a Single Word file

Patuakhali Science & Technology University

Dumki, Patuakhali - 8602

Submitted By

Name      :

Id             :

Reg. No    :

Session    :

Md Mohidul Alam

1902016

08722

2019-20

Submitted To

Professor Dr. Md. Samsuzzaman

Department of Computer and Communication Engineering

Faculty Of Computer Science & Engineering.

Index

Assignment 01 (Based on chapter 05 exercise) --------------------------------------------     03

Assignment 02 (Chapter 01 interactive assignment) -------------------------------------      14

Assignment 03 (Based on chapter 05 exercise) --------------------------------------------     26

Assignment 04 (Related to Chapter 01) -----------------------------------------------------     32

Assignment -01

1. What is the address space in each of the following systems?

- a system with 8-bit addresses

- a system with 16-bit addresses

- a system with 64-bit addresses

Ans: Since each bit can represent 2 numbers 0 and 1, n bits can represent 2n numbers.

- A system with 8-bit addresses can have an address space of

28 addresses = 256 addresses.

- A system with 8-bit addresses can have an address space of

216 addresses = 65,536 addresses.

- A system with 8-bit addresses can have an address space of

264 addresses = 18.446x1018 addresses.

2. An address space has a total of 1,024 addresses. How many bits are needed to represent an address?

Ans: Since 2n addresses need n bits, n addresses need log2n bits.

1,024 addresses need log21024 = log2210 = 10 bits.

3. An address space uses three symbols: 0, 1, and 2 to represent addresses. If each

the address is made of 10 symbols; how many addresses are available in this system?

Ans: Since each symbol can represent 3 numbers 0, 1 and 2, n symbol can represent 3n numbers.

Addresses available with 10 symbols = 310 = 59,049.

4. Change the following IP addresses from dotted-decimal notation to binary notation:

- 114.34.2.8

- 129.14.6.8

- 208.34.54.12

- 238.34.2.1

Ans: 128, 64, 32, 16, 8, 4, 2, 1

- 01110010.00100010.00000010.00001000

- 10000001.00001110.00000110.00001000

- 11010000.00100010.00110110.00001100

- 11101110.00100010.00000010.00000001

5. Change the following IP addresses from dotted-decimal notation to hexadecimal notation:

- 114.34.2.8

- 129.14.6.8

- 208.34.54.12

- 238.34.2.1

Ans: 8, 4, 2, 1

- 01110010.00100010.00000010.00001000 = 0x72220208

- 10000001.00001110.00000110.00001000 = 0x810E0608

- 11010000.00100010.00110110.00001100 = 0xD022360C

- 11101110.00100010.00000010.00000001 = 0xEE220201

6. Change the following IP addresses from hexadecimal notation to binary notation:

- 0x1347FEAB

- 0xAB234102

- 0x0123A2BE

- 0x00001111

Ans: 8, 4, 2, 1

- 00010011.01000111.11111110.10101011

- 10101011.00100011.01000001.00000020

- 00000001.00100011. 10100010. 10111110

- 00000000. 00000000.00010001.00010001

7. How many hexadecimal digits are needed to deﬁne the netid in each of the following classes?

- Class A

- Class B

- Class C

Ans:

- n = 8 bits = 2 hexadecimal digits

- n = 16 bits = 4 hexadecimal digits

- n = 24 bits = 6 hexadecimal digits

8. Change the following IP addresses from binary notation to dotted-decimal notation:

- 01111111 11110000 01100111 01111101

- 10101111 11000000 11111000 00011101

- 11011111 10110000 00011111 01011101

- 11101111 11110111 11000111 00011101

Ans: 128, 64, 32, 16, 8, 4, 2, 1

- 127.240.103.125

- 175.192.248.29

- 223.176.31.93

- 239.247.199.29

9. Find the class of the following IP addresses:

- 208.34.54.12

- 238.34.2.1

- 242.34.2.8

- 129.14.6.8

Ans: Class A 0-127, B 128-191, C 192-223, D 224-239, E 240-255

- Class C

- Class D

- Class E

- Class B

10. Find the class of the following IP addresses:

- 11110111 11110011 10000111 11011101

- 10101111 11000000 11110000 00011101

- 11011111 10110000 00011111 01011101

- 11101111 11110111 11000111 00011101

Ans: Class A 0000-0111, B 1000-1011, C 1100-1101, D 1110, E 1111

- Class E

- Class B

- Class C

- Class D

11. Find the netid and the hostid of the following IP addresses:

- 114.34.2.8

- 132.56.8.6

- 208.34.54.12

- 251.34.98.5

Ans: Class A 0-127, B 128-191, C 192-223, D 224-239, E 240-255

- Class A. Netid 114, Hostid 34.2.8

- Class B. Netid 132.56, Hostid 8.6

- Class C. Netid 208.34.54, Hostid 12

- Class E. Class D and E are not divided into netid and hostid.

12. Find the number of addresses in the range if the ﬁrst address is 14.7.24.0 and the last address is 14.14.34.255.

Ans: We can subtract the ﬁrst address from the last address in base 256. The result is 0.7.10.255 in this base. To ﬁnd the number of addresses in the range, we convert this number to base 10 and add 1 to the result.

Number of addresses = (0 x 2563 + 7 x 2562 + 10 x 2561 +255 x 2560) + 1 = 461,568.

13. If the ﬁrst address in a range is 122.12.7.0 and there are 2048 addresses in the

Range , what is the last address?

Ans: We convert the number of addresses minus 1 to base 256, which is

2048 – 1 = 7 x 256 + 255 = 0.0.7.255.

We then add it to the ﬁrst address to get the last address.

Last address = (122.12.7.0 + 0.0.7.255)256 = 122.12.14.255.

14. Find the result of each operation:

- NOT (22.14.70.34)

- NOT (145.36.12.20)

- NOT (200.7.2.0)

- NOT (11.20.255.255)

Ans:

- 233.241.185.221

- 110.219.243.235

- 55.248.253.255

- 244.235.0.0

15. Find the result of each operation:

- (22.14.70.34) AND (255.255.0.0)

- (12.11.60.12) AND (255.0.0.0)

- (14.110.160.12) AND (255.200.140.0)

- (28.14.40.100) AND (255.128.100.0)

Ans: 110=64+32+8+4+2, 160=128+32, 14=8+4+2, 40=32+8

200=128+64+8, 140=128+8+4, 128=128, 100=64+32+4

- 22.14.0.0

- 12.0.0.0

- 14.72.128.0

- 28.0.32.0

16. Find the result of each operation:

- (22.14.70.34) OR (255.255.0.0)

- (12.11.60.12) OR (255.0.0.0)

- (14.110.160.12) OR (255.200.140.0)

- (28.14.40.100) OR (255.128.100.0)

Ans: 110=64+32+8+4+2, 160=128+32, 14=8+4+2, 40=32+8

200=128+64+8, 140=128+8+4, 128=128, 100=64+32+4

- 255.255.70.34

- 255.11.60.12

- 255.238.172.12

- 255.142.108.100

17. In a class A subnet, we know the IP address of one of the hosts and the subnet

mask as given below: IP Address: 25.34.12.56, Subnet mask: 255.255.0.0.

What is the ﬁrst address (subnet address)? What is the last address?

Ans: Subnet id, nsub = 16. To ﬁnd the ﬁrst address, we keep the leftmost 16 bits and set the rightmost 16 bits all to 0s.

The ﬁrst address (subnet address) is 25.34.0.0.

To ﬁnd the last address, we keep the leftmost 16 bits and set the rightmost 16 bits all to 1s.

The last address is 25.34.255.255.

18. In a class B subnet, we know the IP address of one of the hosts and the subnet

mask as given below: P Address: 131.134.112.66, Subnet mask: 255.255.224.0.

What is the ﬁrst address (subnet address)? What is the last address?

Ans: Subnet id, nsub = 19. To ﬁnd the ﬁrst address, we keep the leftmost 19 bits and set the rightmost 13 bits all to 0s.

The ﬁrst address (subnet address) is 131.134.96.0.

To ﬁnd the last address, we keep the leftmost 19 bits and set the rightmost 13 bits all to 1s.

The last address is 131.134.127.255.

19. In a class C subnet, we know the IP address of one of the hosts and the subnet

mask as given below: IP Address: 202.44.82.16, Subnet mask: 255.255.255.192.

What is the ﬁrst address (subnet address)? What is the last address?

Ans: Subnet id, nsub = 26. To ﬁnd the ﬁrst address, we keep the leftmost 26 bits and set the rightmost 6 bits all to 0s.

The ﬁrst address (subnet address) is 202.44.82.0.

To ﬁnd the last address, we keep the leftmost 26 bits and set the rightmost 6 bits all to 1s.

The last address is 202.44.82.63.

20. Find the subnet mask in each case:

- 1024 subnets in class A

- 256 subnets in class B

- 32 subnets in class C

- 4 subnets in class C

Ans: Subnet id, nsub = n + log2s. Subnet mask has nsub numbers of 1s and (32- nsub) 0s.

- nsub = 8 + log21024 = 8 + log2210 = 18. Subnet mask = 255.255.192.0

- nsub = 16 + log2256 = 16 + log228 = 24. Subnet mask = 255.255.255.0

- nsub = 24 + log232 = 24 + log225 = 29. Subnet mask = 255.255.255.248

- nsub = 24 + log24 = 24 + log222 = 26. Subnet mask = 255.255.255.192

21. In a block of addresses, we know the IP address of one host is 25.34.12.56/16. What is the ﬁrst address (network address) and the last address (limited broadcast address) in this block?

Ans: We use the AND operation to ﬁnd the ﬁrst address (network address).

Known host =        23.    34.  12.  56

Network mask = 255.  255.    0.   0

First address =      23.    34.     0.   0

We use the OR operation to ﬁnd the last address (limited broadcast address).

Known host =                                  23.    34.    12.  56

Complement of network mask =   0.      0.   255.  255

Last address =                                  23.    34.  255.  255

22. In a block of addresses, we know the IP address of one host is 182.44.82.16/26. What is the ﬁrst address (network address) and the last address (limited broadcast address) in this block?

Ans: We use the AND operation to ﬁnd the ﬁrst address (network address).

Known host =      182.    44.    82.   16

Network mask = 255.  255.  255.  192

First address =    182.     44.    82.   0

We use the OR operation to ﬁnd the last address (limited broadcast address).

Known host =                                  182.   44.   82.   16

Complement of network mask =     0.      0.     0.   63

Last address =                                 182.    44.   82.   63

23. In ﬁxed-length subnetting, find the number of 1s that must be added to the mask if the number of desired subnets is _______.

- 2

- 62

- 122

- 250

Ans: Number of 1s that must be added to the mask is, log2s.

- log22 = 1

- log264 = log226 = 6

- log2128 = log227 = 7

- log2256 = log228 = 8

24. An organization is granted the block 16.0.0.0/8. The administrator wants to create 500 ﬁxed-length subnets.

- Find the subnet mask.

- Find the number of addresses in each subnet.

- Find the ﬁrst and the last address in the ﬁrst subnet.

- Find the ﬁrst and the last address in the last subnet (subnet 500).

Ans:

- Subnet mask, nsub = n + log2s = 8 + log2512 = 8 + log229 = 17

- Addresses in each subnet, N = 2^(32-nsub) = 2(32-17) = 215 = 32,768

- First Address = 16.0.0.0. 32,767 = 127x256+255, Last Address = 16.0.127.255

2nd block 16.0.128.0 - 16.0.255.255

3rd block 16.1.0.0 - 16.1.127.255

4th block 16.1.128.0 - 16.1.255.255

- First Address = 16.249.128.0, Last Address = 16.249.255.255

25. An organization is granted block 130.56.0.0/16. The administrator wants to create 1024 subnets.

- Find the subnet mask.

- Find the number of addresses in each subnet.

- Find the ﬁrst and the last address in the ﬁrst subnet.

- Find the ﬁrst and the last address in the last subnet (subnet 1024).

Ans:

- Subnet mask, nsub = n + log2s = 16 + log21024 = 8 + log2210 = 26

- Addresses in each subnet, N = 2^(32-nsub) = 2(32-26) = 26 = 64

- First Address = 130.56.0.0, Last Address = 130.56.0.63

- First Address = 130.56.255.192, Last Address = 130.56.255.255

26. An organization is granted block 211.17.180.0/24. The administrator wants to create 32 subnets.

- Find the subnet mask.

- Find the number of addresses in each subnet.

- Find the ﬁrst and the last address in the ﬁrst subnet.

- Find the ﬁrst and the last address in the last subnet (subnet 32).

Ans:

- Subnet mask, nsub = n + log2s = 24 + log232 = 24 + log225 = 29

- Addresses in each subnet, N = 2^(32-nsub) = 2(32-29) = 23 = 8

- First Address = 211.17.180.0, Last Address = 211.17.180.7

- First Address = 211.17.180.248, Last Address = 211.17.180.255

27. Write the following mask in slash notation (/n):

- 255.255.255.0

- 255.0.0.0

- 255.255.224.0

- 255.255.240.0

Ans:

- 11111111.11111111.11111111.00000000 -> /24

- 11111111.00000000.00000000.00000000 -> /8

- 11111111.11111111.11100000.00000000 -> /19

- 11111111.11111111.11110000.00000000 -> /20

27. Find the range of addresses in the following blocks:

- 123.56.77.32/29

- 200.17.21.128/27

- 17.34.16.0/23

- 180.34.64.64/30

Ans:

- We use the AND operation to ﬁnd the ﬁrst address.

Known host =      123.    56.    77.  32

Network mask = 255.  255.  255.  248

First address =    123.     56.    77.   32

We use the OR operation to ﬁnd the last address.

Known host =                                  123.    56.    77.  32

Complement of network mask =     0.      0.       0.   7

Last address =                                 123.    56.    77.   39

Range of addresses 123.56.77.32 - 123.56.77.39

- We use the AND operation to ﬁnd the ﬁrst address.

Known host =      200.    17.    21.  128

Network mask = 255.  255.  255.  224

First address =    200.    17.    21.  128

We use the OR operation to ﬁnd the last address.

Known host =                                  200.    17.   21.   128

Complement of network mask =     0.      0.      0.   31

Last address =                                 200.    17.    21.   159

Range of addresses 200.17.21.128 - 200.17.21.159

- We use the AND operation to ﬁnd the ﬁrst address.

Known host =       17.     34.    16.   0

Network mask = 255.  255.  254.   0

First address =      17.     34.    16.   0

We use the OR operation to ﬁnd the last address.

Known host =                                  17.     34.    16.   0

Complement of network mask =     0.      0.     1.    255

Last address =                                 17.     34.    17.   255

Range of addresses 17.34.16.0 - 17.34.17.255

- We use the AND operation to ﬁnd the ﬁrst address.

Known host =      180.    34.    64.   64

Network mask = 255.  255.  255.  252

First address =    180.    34.    64.   64

We use the OR operation to ﬁnd the last address.

Known host =                                  180.    34.    64.   64

Complement of network mask =     0.      0.     0.     3

Last address =                                 180.    34.    64.   67

Range of addresses 180.34.64.64 - 180.34.64.67

29. In classless addressing, we know the ﬁrst and the last address in the block. Can we ﬁnd the preﬁx length? If the answer is yes, show the process and give an example.

Ans: We can find prefix length from the first and last addresses in the block.

We know, N = 232-n, where n is the prefix length.

So, prefix length can be found by, n = 32 – log2N

where N is the number of addresses in the block which can be obtained by subtracting the first address from the last address.

Let us consider the block having the first address 180.34.64.64 and the last address 180.34.64.67.

The number of addresses in the block,

N = (180.34.64.67 - 180.34.64.64)256 + 1 = 3 + 1 = 4

Prefix length, n = 32 – log24 = 32 – log222 = 30.

30. In classless addressing, we know the ﬁrst address and the number of addresses in the block. Can we ﬁnd the preﬁx length? If the answer is yes, show the process and give an example.

Ans: We can find prefix length from only the number of addresses in the block.

We know, N = 232-n, where n is the prefix length.

So, prefix length can be found by, n = 32 – log2N

where N is the number of addresses in the block.

Let us consider the block having the first address 17.34.16.0 and the number of addresses 512.

The number of addresses in the block, N = 512

Prefix length, n = 32 – log2512 = 32 – log229 = 23.

31. In classless addressing, can two blocks have the same preﬁx length? Explain.

Ans: Two blocks can have the same prefix length because prefix length only determines the size of the block. But blocks of the same size can reside in different points in the address space.

Let us consider the blocks 17.34.16.0/24 and 17.34.17.0/24.

Both blocks have the same prefix 24 but contain different ranges of addresses.

32. In classless addressing, we know the ﬁrst address and one of the addresses in the block (not necessarily the last address). Can we ﬁnd the preﬁx length? Explain.

Ans: In classless addressing, the preﬁx length cannot be found if we are given only an address in the block even with the first address of the block. The given address can belong to a block with any preﬁx length. In classless addressing, if we need to ﬁnd the block we need either

- preﬁx length or

- both first and last addresses or

- one of the first or last addresses and the number of total addresses.

Let us consider the first address 230.8.24.0 and another address 230.8.24.56.

This may represent any of the blocks from 230.8.24.0/24 to 230.8.24.0/26.

33. An ISP is granted a block of addresses starting with 150.80.0.0/16. The ISP wants to distribute these blocks to 2600 customers as follows:

- The ﬁrst group has 200 medium-size businesses; each needs approximately 128 addresses.

- The second group has 400 small businesses; each needs approximately 16 addresses.

- The third group has 2000 households; each needs 4 addresses.

Design the subblocks and give the slash notation for each subblock. Find out how many addresses are still available after these allocations.

Ans:

Group 1: 200 × 128 = 25,600            n1 = 16 + log2 (65536/25600) = 17

Group 2: 400 × 16 = 6,400                 n2 = 16 + log2 (65536/6400) = 19

Group 3: 2000 × 4 = 8,000                 n3 = 16 + log2 (65536/8000) = 19

- First group starts at 150.80.0.0/17

25,599 = 99 x 256 + 255

First group ends at 150.80.99.255/17

For each customer, nsub = 16 + log2(65536/128) = 16 + log2512 = 16 + log229 = 25

First subblock 150.80.0.0/25 - 150.8.0.127

Last subblock 150.80.99.128/25 -150.80.99.255

- Second group starts at 150.80.100.0/19

6,399 = 24 x 256 + 255

Second group ends at 150.80.124.255/19

For each customer, nsub = 16 + log2(65536/16) = 16 + log24096 = 16 + log2212 = 28

First subblock 150.80.100.0/28

Last subblock 150.80.124.240/28

- Last group starts at 150.80.125.0/19

7,999 = 31 x 256 + 63

Last group ends at 150.80.156.63/19

For each customer, nsub = 16 + log2(65536/4) = 16 + log216384 = 16 + log2214 = 30

First subblock 150.80.125.0/30

Last subblock 150.80.156.60/30

Total granted addresses, N = 232-16 = 65,536

Total addresses used = 25,600 + 6,400 + 8,000 = 40,000

Addresses still available = 65,536 – 40,000 = 25,536

The first address in this range is 150.80.156.64 and last address is 150.80.255.255.

34. An ISP is granted a block of addresses starting with 120.60.4.0/20. The ISP wants to distribute these blocks to 100 organizations with each organization receiving 8 addresses only. Design the subblocks and give the slash notation for each subblock. Find out how many addresses are still available after these allocations.

Ans: Total addresses granted, N = 232-20 = 4096

Addresses needed: 100 × 8 = 800            nsub = 20 + log2 (4096/800) = 22

Address allocation starts at 120.60.4.0/22

799 = 3 x 256 + 31

Address allocation ends at 120.60.7.31/22

For each customer, nsub = 20 + log2(4096/8) = 20 + log2512 = 20 + log229 = 29

First subblock 120.60.4.0/29 - 120.60.4.7/29

Last subblock 120.60.7.24/29 - 120.60.7.31/29

Addresses still available = 4096 – 800 = 3,296

35. An ISP has a block of 1024 addresses. It needs to divide the addresses to 1024 customers. Does it need subnetting? Explain your answer.

Ans: An ISP does not need subnetting to distribute 1024 addresses to 1024 customers because each customer only needs one address not a block of addresses. Although an ISP can do subnetting using the /32 prefix as it defines 1 address as a block. But it will require extra hardware which makes subnetting in this case inefficient.

Assignment-02
 
CIRCUIT SWITCHING

Consider the circuit-switched network shown in the figure below, with circuit switches A, B, C, and D. Suppose there are 11 circuits between A and B, 13 circuits between B and C, 12 circuits between C and D, and 18 circuits between D and A.

- What is the maximum number of connections that can be ongoing in the network at any one time ?

Ans: The maximum number of connections that can be ongoing at any one time is the sum of all circuits, which happens when 17 connections go from A to B, 14 connections go from B to C, 15 connections go from C to D, and 15 connections go from D to A. This sum is 61.

- Suppose that these maximum number of connections are all ongoing. What happens when another call connection request arrives to the network, will it be accepted? Answer Yes or No

Ans: No, it will be blocked because there are no free circuits.

- Suppose that every connection requires 2 consecutive hops, and calls are connected clockwise. For example, a connection can go from A to C, from B to D, from C to A, and from D to B. With these constraints, what is the is the maximum number of connections that can be ongoing in the network at any one time?

Ans : There can be a maximum of 29 connections. Consider routes A->C and C->A, sum the bottleneck links, consider any leftover capacity that would allow for B->D and D->B connections, and compare that value to the equivalent of B->D and D->B.

- Suppose that 14 connections are needed from A to C, and 17 connections are needed from B to D. Can we route these calls through the four links to accommodate all 31 connections? Answer Yes or No

Ans : Using our answer from question 4, the sum of our needed connections is 31, and we have 29 available connections, so it is NOT possible.

QUANTITATIVE COMPARISON OF PACKET SWITCHING AND CIRCUIT SWITCHING

This question requires a little bit of background in probability (but we'll try to help you though it in the solutions). Consider the two scenarios below:

- A circuit-switching scenario in which Ncs users, each requiring a bandwidth of 25 Mbps, must share a link of capacity 100 Mbps.

- A packet-switching scenario with Nps users sharing a 100 Mbps link, where each user again requires 25 Mbps when transmitting, but only needs to transmit 10 percent of the time.

Round your answer to two decimals after leading zeros

- When circuit switching is used, what is the maximum number of users that can be supported?

Ans : When circuit switching is used, at most 5 users can be supported. This is because each circuit-switched user must be allocated its 20 Mbps bandwidth, and there is 100 Mbps of link capacity that can be allocated.

- Suppose packet switching is used. If there are 9 packet-switching users, can this many user be supported under circuit-switching? Yes or No.

Ans : No. Under circuit switching, the 9 users would each need to be allocated 20 Mbps, for an aggregate of 180 Mbps - more than the 100 Mbps of link capacity available.

- Suppose packet switching is used. What is the probability that a given (specific) user is transmitting, and the remaining users are not transmitting?

Ans : The probability that a given (specific) user is busy transmitting, which we'll denote p, is just the fraction of time it is transmitting, i.e. 0.3. The probability that one specific other user is not busy is (1-p), and so the probability that all of the other Nps-1 users are not transmitting is (1-p)Nps-1. Thus the probability that one specific user is transmitting and the remaining users are not transmitting is p*(1-p)Nps-1, which has the numerical value of 0.017.

- Suppose packet switching is used. What is the probability that one user (any one among the 9 users) is transmitting, and the remaining users are not transmitting?

Ans : Suppose packet switching is used. What is the probability that one user (any one among the 9 users) is transmitting, and the remaining users are not transmitting?

- When one user is transmitting, what fraction of the link capacity will be used by this user? Write your answer as a decimal.

Ans : This user will be transmitting at a rate of 20 Mbps over the 100 Mbps link, using a fraction 0.2 of the link's capacity when busy.

- What is the probability that any 3 users (of the total 9 users) are transmitting and the remaining users are not transmitting?

Ans : The probability that 3 specific users of the total 9 users are transmitting and the other 6 users are idle is p3(1-p)6. Thus the probability that any 4 of the 7 users are busy is choose(9, 3) * p3(1-p)6, where choose(9, 3) is the (9, 3) coefficient of the binomial distribution). The numerical value of this probability is 0.27.

- What is the probability that more than 5 users are transmitting?

Ans : The probability that more than 5 users of the total 9 users are transmitting is Σ i=6,9 choose(9, i) * pi(1-p)9 - i. The numerical value of this probability is 0.025. Note that 5 is the maximum number of users that can be supported using circuit switching. With packet switching, nearly twice as many users (9) are supported with a small probability that more than 5 of these packet-switching users are busy at the same time.

### CAR - CARAVAN ANALOGY

Consider the figure below, adapted from Figure 1.17 in the text, which draws the analogy between store-and-forward link transmission and propagation of bits in packet along a link, and cars in a caravan being serviced at a toll booth and then driving along a road to the next tollbooth.

Suppose the caravan has 20 cars, and that the tollbooth services (that is, transmits) a car at a rate of one car per 2 seconds. Once receiving serving a car proceeds to the next tool both, which is 500 kilometers away at a rate of 20 kilometers per second. Also assume that whenever the first car of the caravan arrives at a tollbooth, it must wait at the entrance to the tollbooth until all of the other cars in its caravan have arrived, and lined up behind it before being serviced at the toll booth. (That is, the entire caravan must be stored at the tollbooth before the first car in the caravan can pay its toll and begin driving towards the next tollbooth).

- Once a car enters service at the tollbooth, how long does it take until it leaves service?

Ans :  Service time is 5 seconds.

- How long does it take for the entire caravan to receive service at the tollbooth (that is the time from when the first car enters service until the last car leaves the tollbooth)?

Ans : It takes 50 seconds to service every car, (10 cars * 5 seconds per car).

- Once the first car leaves the tollbooth, how long does it take until it arrives at the next tollbooth?

Ans : It takes 25 seconds to travel to the next toll booth (500 km / 20 km/s).

- Once the last car leaves the tollbooth, how long does it take until it arrives at the next tollbooth?

Ans :  Just like in the previous question, it takes 25 seconds, regardless of the car.

- Once the first car leaves the tollbooth, how long does it take until it enters service at the next tollbooth?

Ans : It takes 70 seconds until the first car gets serviced at the next toll booth (10-1 cars * 5 seconds per car + 500 km / 20 km/s).

- Are there ever two cars in service at the same time, one at the first toll booth and one at the second toll booth? Answer Yes or No

Ans : No, because cars can't get service at the next tollbooth until all cars have arrived.

- Are there ever zero cars in service at the same time, i.e., the caravan of cars has finished at the first toll both but not yet arrived at the second tollbooth? Answer Yes or No

Ans : Yes, one notable example is when the last car in the caravan is serviced but is still travelling to the next toll booth; all other cars have to wait until it arrives, thus no cars are being serviced.

### COMPUTING THE ONE-HOP TRANSMISSION DELAY

Consider the figure below, in which a single router is transmitting packets, each of length L bits, over a single link with transmission rate R Mbps to another router at the other end of the link.

Suppose that the packet length is L= 4000 bits, and that the link transmission rate along the link to router on the right is R = 1 Mbps.

Round your answer to two decimals after leading zeros

- What is the transmission delay?

Ans : The transmission delay = L/R = 8000 bits / 1000000000 bps = 8.00E-6 seconds .

2. What is the maximum number of packets per second that can be transmitted by this link?

Ans : The number of packets that can be transmitted in a second into the link = R / L = 1000000000 bps / 8000 bits = 125000 packets.

### QUEUING DELAY

Consider the queuing delay in a router buffer, where the packet experiences a delay as it waits to be transmitted onto the link. The length of the queuing delay of a specific packet will depend on the number of earlier-arriving packets that are queued and waiting for transmission onto the link. If the queue is empty and no other packet is currently being transmitted, then our packet’s queuing delay will be zero. On the other hand, if the traffic is heavy and many other packets are also waiting to be transmitted, the queuing delay will be long.

Assume a constant transmission rate of R = 300000 bps, a constant packet-length L = 4600 bits, and a is the average rate of packets/second. Traffic intensity I = La/R, and the queuing delay is calculated as I(L/R)(1 - I) for I < 1.

1. In practice, does the queuing delay tend to vary a lot? Answer with Yes or No

Ans : Yes, in practice, queuing delay can vary significantly. We use the above formulas as a way to give a rough estimate, but in a real-life scenario it is much more complicated. 

2. Assuming that a = 28, what is the queuing delay? Give your answer in milliseconds (ms)

Ans : Queuing Delay = I(L/R)(1 - I) * 1000 = 0.0887*(1900/600000)*(1-0.0887) * 1000 = 0.256 ms.

3. Assuming that a = 52, what is the queuing delay? Give your answer in milliseconds (ms)

Ans : Queuing Delay = I(L/R)(1 - I) * 1000 = 0.1647*(1900/600000)*(1-0.1647) * 1000 = 0.4357 ms. 


4. Assuming the router's buffer is infinite, the queuing delay is 0.4357 ms, and 1218 packets arrive. How many packets will be in the buffer 1 second later?

Ans : Packets left in buffer = a - floor(1000/delay) = 1218 - floor(1000/0.4357) = 0 packets

5. If the buffer has a maximum size of 563 packets, how many of the 1218 packets would be dropped upon arrival from the previous question?

Ans : Packets dropped = packets - buffer size = 1218 - 563 = 655 dropped packets.

### COMPUTING END-END DELAY (TRANSMISSION AND PROPAGATION DELAY)

Consider the figure below, with three links, each with the specified transmission rate and link length.

Assume the length of a packet is 8000 bits. The speed of light propagation delay on each link is 3x10^8 m/sec

Round your answer to two decimals after leading zeros

- What is the transmission delay of link 1?

Ans : Link 1 transmission delay = L/R = 8000 bits / 10 Mbps = 0.0008 seconds

- What is the propogation delay of link 1?

Ans : Link 1 propagation delay = d/s = ()3 Km) * 1000 / 3*10^8 m/sec = 1.00E-5 seconds

3. What is the total delay of link 1?

Ans : Link 1 total delay = d_t + d_p = 0.0008 seconds + 1.00E-5 seconds = 0.00081 seconds

4. What is the transmission delay of link 2?

Ans : Link 2 transmission delay = L/R = 8000 bits / 100 Mbps = 8.00E-5 seconds 

5. What is the propogation delay of link 2?

Ans : Link 2 propagation delay = d/s = ()500 Km) * 1000 / 3*10^8 m/sec = 0.0017 seconds

6. What is the total delay of link 2?

Ans : Link 2 total delay = d_t + d_p = 8.00E-5 seconds + 0.0017 seconds = 0.0017 seconds

7. What is the transmission delay of link 3?

Ans : Link 3 transmission delay = L/R = 8000 bits / 1000 Mbps = 8.00E-6 seconds

8. What is the propogation delay of link 3?

Ans : Link 3 propagation delay = d/s = ()3 Km) * 1000 / 3*10^8 m/sec = 1.00E-5 seconds

9. What is the total delay of link 3?

Ans : Link 3 total delay = d_t + d_p = 8.00E-6 seconds + 1.00E-5 seconds = 1.80E-5 seconds

10. What is the total delay?

Ans : The total delay = d_L1 + d_L2 + d_L3 = 0.00081 seconds + 0.0017 seconds + 1.80E-5 seconds = 0.0026 second.

### END TO END THROUGHPUT AND BOTTLENECK LINKS

Consider the scenario shown below, with four different servers connected to four different clients over four three-hop paths. The four pairs share a common middle hop with a transmission capacity of R = 400 Mbps. The four links from the servers to the shared link have a transmission capacity of RS = 80 Mbps. Each of the four links from the shared middle link to a client has a transmission capacity of RC = 30 Mbps.

You might want to review Figure 1.20 in the text before answering the following questions

1. What is the maximum achievable end-end throughput (in Mbps) for each of four client-to-server pairs, assuming that the middle link is fairly shared (divides its transmission rate equally)?
Ans: The maximum achievable end-end throughput is the capacity of the link with the minimum capacity, which is 20 Mbps.

2. Which link is the bottleneck link? Format as Rc, Rs, or R
Ans: The bottleneck link is the link with the smallest capacity between RS, RC, and R/4. The bottleneck link is Rc.

3. Assuming that the servers are sending at the maximum rate possible, what are the link utilizations for the server links (RS)? Answer as a decimal
Ans: The server's utilization = Rbottleneck / RS = 20 / 50 = 0.4

4. Assuming that the servers are sending at the maximum rate possible, what are the link utilizations for the client links (RC)? Answer as a decimal
Ans: The client's utilization = Rbottleneck / RC = 20 / 20 = 1

5. Assuming that the servers are sending at the maximum rate possible, what is the link utilizations for the shared link (R)? Answer as a decimal

Ans: The shared link's utilization = Rbottleneck / (R / 4) = 20 / (300 / 4) = 0.27

THE IP STACK AND PROTOCOL LAYERING

In the scenario below, imagine that you're sending an http request to another machine somewhere on the network.

1. What layer in the IP stack best corresponds to the phrase: 'bits live on the wire'
Ans: The given phrase corresponds to the Physical Layer.
 
2. What layer in the IP stack best corresponds to the phrase: 'passes frames from one node to another across some medium'
Ans: The given phrase corresponds to the Link Layer.

3. What layer in the IP stack best corresponds to the phrase: 'moves datagrams from the source host to the destination host'
Ans: The given phrase corresponds to the Network Layer.

4. What layer in the IP stack best corresponds to the phrase: 'handles messages from a variety of network applications'
Ans: The given phrase corresponds to the Application Layer.

5. What layer in the IP stack best corresponds to the phrase: 'handles the delivery of segments from the application layer, may be reliable or unreliable'
Ans: The given phrase corresponds to the Transport Layer.

6. What layer corresponds to box 1?
Ans: Box 1 is the Application Layer.

7. What layer corresponds to box 2?
Ans: Box 2 is the Transport Layer.

8. What layer corresponds to box 3?
Ans: Box 3 is the Network Layer.

9. What layer corresponds to box 4?
Ans: Box 4 is the Link Layer.

10. What layer corresponds to box 5?
Ans: Box 5 is the Physical Layer.

11. What layer corresponds to box 6?
Ans: Box 6 is the Physical Layer.

12. What layer corresponds to box 7?
Ans: Box 7 is the Link Layer.

13. What layer corresponds to box 8?
Ans: Box 8 is the Physical Layer.

14. What layer corresponds to box 9?
Ans: Box 9 is the Link Layer.

15. What layer corresponds to box 10?
Ans: Box 10 is the Network Layer.

16. What layer corresponds to box 11?

Ans: Box 11 is the Physical Layer.


17. What layer corresponds to box 12?
Ans: Box 12 is the Link Layer.

18. What layer corresponds to box 13?
Ans: Box 13 is the Network Layer.

19. What layer corresponds to box 14?
Ans: Box 14 is the Transport Layer.

20. What layer corresponds to box 15?
Ans: Box 15 is the Application Layer.

Assignment-03

R1. What is the difference between a host and an end system? List several different types of end systems. Is a Web server an end system?

Ans:   There is no difference. Throughout this text, the words “host” and “end system” are used interchangeably. End systems include PCs, workstations, Web servers, mail servers, PDAs, Internet-connected game consoles, etc.

R2. The word protocol is often used to describe diplomatic relations. How does Wikipedia describe diplomatic protocol?

Ans: From Wikipedia: Diplomatic protocol is commonly described as a set of international courtesy rules. These well-established and time-honored rules have made it easier for nations and people to live and work together. Part of protocol has always been the acknowledgment of the hierarchical standing of all present. Protocol rules are based on the principles of civility.

R3. Why are standards important for protocols?

Ans: Standards are important for protocols so that people can create networking systems and products that interoperate.

R4. List six access technologies. Classify each one as home access, enterprise

access, or wide-area wireless access.

Ans: 1. Dial-up modem over telephone line: home;

2. DSL over telephone line: home or small office;

3. Cable to HFC: home;

4. 100 Mbps switched Ethernet: enterprise;

5. Wifi (802.11): home and enterprise

6. 3G and 4G: wide-area wireless.

R5. Is HFC transmission rate dedicated or shared among users? Are collisions

possible in a downstream HFC channel? Why or why not?

Ans: HFC bandwidth is shared among the users. On the downstream channel, all packets emanate from a single source, namely, the head end. Thus, there are no collisions in the downstream channel.

R6. List the available residential access technologies in your city. For each type of access, provide the advertised downstream rate, upstream rate, and monthly price.

Ans: In most American cities, the current possibilities include: dial-up; DSL; cable modem; fiber to-the-home.

R7. What is the transmission rate of Ethernet LANs?

Ans: Ethernet LANs have transmission rates of 10 Mbps, 100 Mbps, 1 Gbps and 10 Gbps.

R8. What are some of the physical media that Ethernet can run over?

Ans: Today, Ethernet most commonly runs over twisted-pair copper wire. It also can run over fibers optic links

R9. Dial-up modems, HFC, DSL and FTTH are all used for residential access. For each of these access technologies, provide a range of transmission rates and comment on whether the transmission rate is shared or dedicated.

Ans: Dial up modems: up to 56 Kbps, bandwidth is dedicated; ADSL: up to 24 Mbps downstream and 2.5 Mbps upstream, bandwidth is dedicated; HFC, rates up to 42.8 Mbps and upstream rates of up to 30.7 Mbps, bandwidth is shared. FTTH: 2-10Mbps upload; 10-20 Mbps download; bandwidth is not shared.

R10. Describe the most popular wireless Internet access technologies today. Compare and contrast them.

Ans: There are two popular wireless Internet access technologies today:

a) Wifi (802.11) In a wireless LAN, wireless users transmit/receive packets to/from an base station (i.e., wireless access point) within a radius of few tens of meters. The base station is typically connected to the wired Internet and thus serves to connect wireless users to the wired network.

b) 3G and 4G wide-area wireless access networks. In these systems, packets are transmitted over the same wireless infrastructure used for cellular telephony, with the base station thus being managed by a telecommunications provider. This provides wireless access to users within a radius of tens of kilometers of the base station.

R11. Suppose there is exactly one packet switch between a sending host and a receiving host. The transmission rates between the sending host and the switch and between the switch and the receiving host are R1 and R2, respectively. Assuming that the switch uses store-and-forward packet switching, what is the total end-to-end delay to send a packet of length L? (Ignore queuing, propagation delay, and processing delay.)

Ans: At time t0 the sending host begins to transmit. At time t1 = L/R1, the sending host completes transmission and the entire packet is received at the router (no propagation delay). Because the router has the entire packet at time t1, it can begin to transmit the packet to the receiving host at time t1. At time t2 = t1 + L/R2, the router completes transmission and the entire packet is received at the receiving host (again, no propagation delay). Thus, the end-to-end delay is L/R1+ L/R

R12. What advantage does a circuit-switched network have over a packet-switched network? What advantages does TDM have over FDM in a circuit-switched network?

Ans: A circuit-switched network can guarantee a certain amount of end-to-end bandwidth for the duration of a call. Most packet-switched networks today (including the Internet) cannot make any end-to-end guarantees for bandwidth. FDM requires sophisticated analog hardware to shift signal into appropriate frequency bands.

R13. Suppose users share a 2 Mbps link. Also suppose each user transmits

continuously at 1 Mbps when transmitting, but each user transmits only 20 percent

of the time. (See the discussion of statistical multiplexing in Section 1.3.)

- When circuit switching is used, how many users can be supported?

- For the remainder of this problem, suppose packet switching is used. Why will there be essentially no queuing delay before the link if two or fewer users transmit at the same time? Why will there be a queuing delay if three users transmit at the same time?

- Find the probability that a given user is transmitting.

- Suppose now there are three users. Find the probability that at any given time, all three users are transmitting simultaneously. Find the fraction of time during which the queue grows.

Ans:

- 2 users can be supported because each user requires half of the link bandwidth.

- Since each user requires 1Mbps when transmitting, if two or fewer users transmit simultaneously, a maximum of 2Mbps will be required. Since the available bandwidth of the shared link is 2Mbps, there will be no queuing delay before the link. Whereas, if three users transmit simultaneously, the bandwidth required will be 3Mbps which is more than the available bandwidth of the shared link. In this case, there will be queuing delay before the link.

- Probability that a given user is transmitting = 0.2

- Probability that all three users are transmitting simultaneously=. Since the queue grows when all the users are transmitting, the fraction of time during which the queue grows (which is equal to the probability that all three users are transmitting simultaneously) is 0.008.

R14. Why will two ISPs at the same level of the hierarchy often peer with each

other? How does an IXP earn money?

Ans:

If the two ISPs do not peer with each other, then when they send traffic to each other they have to send the traffic through a provider ISP (intermediary), to which they have to pay for carrying the traffic. By peering with each other directly, the two ISPs can reduce their payments to their provider ISPs. An Internet Exchange Points (IXP) (typically in a standalone building with its own switches) is a meeting point where multiple ISPs can connect and/or peer together. An ISP earns its money by charging each of the the ISPs that connect to the IXP a relatively small fee, which may depend on the amount of traffic sent to or received from the IXP

R15. Some content providers have created their own networks. Describe Google’s network. What motivates content providers to create these networks

Ans:

Google's private network connects together all its data centers, big and small. Traffic between the Google data centers passes over its private network rather than over the public Internet. Many of these data centers are located in, or close to, lower tier ISPs. Therefore, when Google delivers content to a user, it often can bypass higher tier ISPs. What motivates content providers to create these networks? First, the content provider has more control over the user experience, since it has to use few intermediary ISPs. Second, it can save money by sending less traffic into provider networks. Third, if ISPs decide to charge more money to highly profitable content providers (in countries where net neutrality doesn't apply), the content providers can avoid these extra payments.

R16. Consider sending a packet from a source host to a destination host over a

fixed route. List the delay components in the end-to-end delay. Which of these

delays are constant and which are variable?

Ans: The delay components are processing delays, transmission delays, propagation delays, and

queuing delays. All of these delays are fixed, except for the queuing delays, which are variable.

R18. How long does it take a packet of length 1,000 bytes to propagate over a link of distance 2,500 km, propagation speed 2.5 · 108 m/s, and transmission rate 2 Mbps? More generally, how long does it take a packet of length L to propagate over a link of distance d, propagation speed s, and transmission rate R bps? Does this delay depend on packet length? Does this delay depend on transmission rate?

Ans: 10msec; d/s; no; no

R19. Suppose Host A wants to send a large file to Host B. The path from Host A to

Host B has three links, of rates R1 = 500 kbps, R2 = 2 Mbps, and R3 = 1 Mbps.

a) Assuming no other traffic in the network, what is the throughput for the file

transfer?

b) Suppose the file is 4 million bytes. Dividing the file size by the throughput,

roughly how long will it take to transfer the file to Host B?

c) Repeat (a) and (b), but now with R2 reduced to 100 kbps.

Ans:

a) 500 kbps

b) 64 seconds

c) 100kbps; 320 second

R20. Suppose end system A wants to send a large file to end system B. At a very high level, describe how end system A creates packets from the file. When one of these packets arrives to a packet switch, what information in the packet does the switch use to determine the link onto which the packet is forwarded? Why is packet switching in the Internet analogous to driving from one city to another and asking directions along the way?

Ans: End system A breaks the large file into chunks. It adds header to each chunk, thereby generating multiple packets from the file. The header in each packet includes the IP address of the destination (end system B). The packet switch uses the destination IP address in the packet to determine the outgoing link. Asking which road to take is analogous to a packet asking which outgoing link it should be forwarded on, given the packet’s destination address.

R21. Visit the Queuing and Loss applet at the companion Web site. What is the maximum emission rate and the minimum transmission rate? With those rates, what is the traffic intensity? Run the applet with these rates and determine how long it takes for packet loss to occur. Then repeat the experiment a second time and determine again how long it takes for packet loss to occur. Are the values different? Why or why not?

Ans:

The maximum emission rate is 500 packets/sec and the maximum transmission rate is 350 packets/sec. The corresponding traffic intensity is 500/350 =1.43 > 1. Loss will eventually occur for each experiment; but the time when loss first occurs will be different from one experiment to the next due to the randomness in the emission process.

R22. List five tasks that a layer can perform. Is it possible that one (or more) of these tasks could be performed by two (or more) layers?

Ans:

Five generic tasks are error control, flow control, segmentation and reassembly, multiplexing, and connection setup. Yes, these tasks can be duplicated at different layers. For example, error control is often provided at more than one layer.

R23. What are the five layers in the Internet protocol stack? What are the principal responsibilities of each of these layers?
Ans:

The five layers in the Internet protocol stack are – from top to bottom – the application layer, the transport layer, the network layer, the link layer, and the physical layer. The principal responsibilities are outlined in Section 1.5.1.

R24. What is an application-layer message? A transport-layer segment? A network-layer datagram? A link-layer frame?

Ans:

Application-layer message: data which an application wants to send and passed onto the transport layer; transport-layer segment: generated by the transport layer and encapsulates application-layer message with transport layer header; network-layer datagram: encapsulates transport-layer segment with a network-layer header; link-layer frame: encapsulates networklayer datagram with a link-layer header.

R25. Which layers in the Internet protocol stack does a router process? Which layers does a link-layer switch process? Which layers does a host process?

Ans: Routers process network, link and physical layers (layers 1 through 3). (This is a little bit of a white lie, as modern routers sometimes act as firewalls or caching components, and process Transport layer as well.) Link layer switches process link and physical layers (layers 1 through2). Hosts process all five layers.

Assignment -04

## Chapter 1 (Knowledge Checks)

## 1/ WHAT IS THE INTERNET?

Which of the following descriptions below correspond to a "nuts-and-bolts" view of the Internet? Select one or more of the answers below that are correct. [Hint: more than one of answers below are correct].

A platform for building network applications.

✅A collection of hardware and software components executing protocols that define the format and the order of messages exchanged between two or more communicating entities, as well as the actions taken on the transmission and/or receipt of a message or other event.

✅A "network of networks".

✅A collection of billions of computing devices, and packet switches interconnected by links.

A place I go for information, entertainment, and to communicate with people

## 2. WHAT IS THE INTERNET (2)?

Which of the following descriptions below correspond to a "services" view of the Internet? Select one or more of the answers below below that are correct below that are correct.  [Hint: more than one of answers below are correct].

A "network of networks".

✅A place I go for information, entertainment, and to communicate with people.

✅A platform for building network applications.

A collection of hardware and software components executing protocols that define the format and the order of messages exchanged between two or more communicating entities, as well as the actions taken on the transmission and/or receipt of a message or other event.

A collection of billions of computing devices, and packet switches interconnected by links.

## WHAT IS A PROTOCOL?

Which of the following human scenarios involve a protocol (recall: "Protocols define the format, order of messages sent and received among network entities, and actions taken on message transmission, receipt")? Select one or more answers below that are correct. Hint: more than one of answers below are correct.

✅Two people introducing themselves to each other.

✅One person asking, and getting, the time to/from another person.

A person sleeping.

A person reading a book.

✅A student raising her/his hand to ask a really insightful question, followed by the teaching acknowledging the student, listening carefully to the question, and responding with a clear, insightful answer.  And then thanking the student for the question, since teachers love to get questions.

2/ The Network Edge

1. Ethernet

Wired. Up to 100's of Mbps downstream per link.

2. 802.11 WiFi

Wireless. 10's to 100's of Mbps per device

3. Cable access network

Wired. Up to 10's to 100's of Mbps downstream per user.

4. Digital Subscriber Line

Wired. Up to 10's of Mbps downstream per user.

5. 4G cellular LTE

Wireless. Up to 10's Mbps per device

## 2/ LINK TRANSMISSION CHARACTERISTICS.

Which of the following physical layer technologies has the highest transmission rate and lowest bit error rate in practice?

✅Fiber optic cable

4G/5G cellular

Satellite channel

Twisted pair (e.g., CAT5, CAT6)

Coaxial cable

802.11 WiFi Channel

3/ The Network Core

## 1/ ROUTING VERSUS FORWARDING.

Choose one the following two definitions that makes the correct distinction between routing versus forwarding.

Routing is the local action of moving arriving packets from router’s input link to appropriate router output link, while forwarding is the global action of determining the source-destination paths taken by packets.

✅Forwarding is the local action of moving arriving packets from router’s input link to appropriate router output link, while routing is the global action of determining the source-destination paths taken by packets.

## 2/ PACKET SWITCHING VERSUS CIRCUIT SWITCHING (1).

Which of the characteristics below are associated with the technique of packet switching? Select all correct answers. [Hint: more than one of the answers is correct].

✅Congestion loss and variable end-end delays are possible with this technique.

✅Data may be queued before being transmitted due to other user’s data that’s also queueing for transmission.

✅This technique is used in the Internet.

Frequency Division Multiplexing (FDM) and Time Division Multiplexing (TDM) are two approaches for implementing this technique.

Reserves resources needed for a call from source to destination.

This technique was the basis for the telephone call switching during the 20th century and into the beginning of this current century.

✅Resources are used on demand, not reserved in advance.

## 3/ PACKET SWITCHING VERSUS CIRCUIT SWITCHING (2).

Which of the characteristics below are associated with the technique of circuit switching? Select all correct answers. [Hint: more than one of the answers is correct].

Congestion loss and variable end-end delays are possible with this technique.

✅Reserves resources needed for a call from source to destination.

Data may be queued before being transmitted due to other user’s data that’s also queueing for transmission.

This technique is used in the Internet.

✅This technique was the basis for the telephone call switching during the 20th century and into the beginning of this current century.

Resources are used on demand, not reserved in advance.

✅Frequency Division Multiplexing (FDM) and Time Division Multiplexing (TDM) are two approaches for implementing this technique

## 4/ HOW MANY CALLS CAN BE CARRIED?

Consider the circuit-switched network shown in the figure below, with  four circuit switches A, B, C, and D. Suppose there are 20 circuits between A and B, 19 circuits between B and C, 15 circuits between C and D, and 16 circuits between D and A.

What is the maximum number of connections that can be ongoing in the network at any one time?

[Note: you can find more questions like this one here.

16

31

✅70

39

20

## 5/ TRYING OUT TRACEROUTE.

Perform a traceroute from your computer (on whatever network you happen to be on) to gaia.cs.umass.edu. Use traceroute (on Mac terminal) or tracert (on Windows command line) or tracepath (on a Linux command line). Enter the missing part of the name of the router just before the host gaia.cs.umass.edu is reached:

??.cs.umass.edu

Note: Routing may change, so the answer here may not be correct anymore.  Also, if you are a Verizon user, there are known problems using traceroute with Verizon - if traceroute shows you two hops only to gaia.cs.umass.edu or any destination, skip this question.

= nscs1bbs1

## 6/ WHAT IS A NETWORK OF NETWORKS?

When we say that the Internet is a “network of networks,” we mean? Check all that apply (hint: check two or more).

✅The Internet is made up of a lot of different networks that are interconnected to each other.

✅The Internet is made up of access networks at the edge, tier-1 networks at the core, and interconnected regional and content provider networks as well.

The Internet is the largest network ever built.

The Internet is the fastest network ever built.

## 7/ PACKET SWITCHING OR CIRCUIT-SWITCHING?

Consider a scenario in which 5 users are being multiplexed over a channel of 10 Mbps.  Under the various scenarios below, match the scenario to whether circuit switching or packet switching is better.

Each user generates traffic at an average rate of 2.1 Mbps, generating traffic at a rate of 15 Mbps when transmitting: Neither works well in this overload scenario

Each user generates traffic at an average rate of 2 Mbps, generating traffic at a rate of 2 Mbps when transmitting.: Circuit switching

Each user generates traffic at an average rate of 0.21 Mbps, generating traffic at a rate of 15 Mbps when transmitting.: Packet switching

4/ Performance: Delay, Loss and Throughput in Computer Networks

## 1/ COMPONENTS OF PACKET DELAY.

Match the description of each component of packet delay to its name in the pull down list.

> Time needed to perform an integrity check, lookup packet information in a local table and move the packet from an input link to an output link in a router.: Processing delay

> Time spent waiting in packet buffers for link transmission.: Queueing delay

> Time spent transmitting packets bits into the link.: Transmission delay

> Time need for bits to physically propagate through the transmission medium from end one of a link to the other.: Propagation delay

## 2/ COMPUTING PACKET TRANSMISSION DELAY(1).

Suppose a packet is L = 1500 bytes long (one byte = 8 bits), and link transmits at R = 1 Gbps (i.e., a link can transmit bits 1,000,000,000 bits per second).  What is the transmission delay for this packet? [Note: you can find more problems like this one here.]

.0015 secs

.0000015 secs

666,666 secs

✅.000012 secs

.00012 secs

## 3/ COMPUTING PACKET TRANSMISSION DELAY (2).

Suppose a packet is L = 1200 bytes long (one byte = 8 bits), and link transmits at R = 100 Mbps (i.e., a link can transmit bits 100,000,000 bits per second).  What is the transmission delay for this packet? [Note: you can find more problems like this one here.]

8,333 secs

✅.000096 secs

.000015 secs

.00096 secs

.0012 secs

## 4/ COMPUTING PACKET TRANSMISSION DELAY (3).

Consider the network shown in the figure below, with three links, each with the specified transmission rate and link length. Assume the length of a packet is 8000 bits.

What is the transmission delay at link 2?  [Note: you can find more problems like this one here.]

12.5 secs

✅8 x 10^(-5) secs

12,500 secs

100 secs

.00096 secs

## 5/ COMPUTING PROPAGATION DELAY.

Consider the network shown in the figure below, with three links, each with the specified transmission rate and link length. Assume the length of a packet is 8000 bits. The speed of light propagation delay on each link is 3x10^8 m/sec

What is the propagation delay at (along) link 2?

.33 secs

3 x 10^8 secs

✅.0033 secs

3 secs

## 6/ COMPUTING THROUGHPUT: A SIMPLE SCENARIO.

What is the maximum throughput achievable between sender and receiver in the scenario shown below?

10 Mbps

✅1.5 Mbps

11.5 Mbps

## 7/ COMPUTING THROUGHPUT.

Consider the scenario shown below, with four different servers connected to four different clients over four three-hop paths. The four pairs share a common middle hop with a transmission capacity of R = 300 Mbps. The four links from the servers to the shared link have a transmission capacity of RS = 50 Mbps. Each of the four links from the shared middle link to a client has a transmission capacity of RC = 90 Mbps.

What is the maximum achievable end-end throughput (an integer value, in Mbps) for each of four client-to-server pairs, assuming that the middle link is fairly shared (divides its transmission rate equally) and all servers are trying to send at their maximum rate?

Your answer: 50 Mbps

## 8/ COMPUTING UTLILIZATION (1).

Consider the scenario shown below, with four different servers connected to four different clients over four three-hop paths. The four pairs share a common middle hop with a transmission capacity of R = 300 Mbps. The four links from the servers to the shared link have a transmission capacity of RS = 50 Mbps. Each of the four links from the shared middle link to a client has a transmission capacity of RC = 90 Mbps.

Assuming that the servers are all sending at their maximum rate possible, what are the link utilizations for the server links (with transmission capacity RS)? Enter your answer in a decimal form of 1.00 (if the utilization is 1) or 0.xx (if the utilization is less than 1, rounded to the closest xx).

Your answer: The utilization of the server links is: 1.00

## 9/ COMPUTING UTILIZATION (2).

Consider the scenario shown below, with four different servers connected to four different clients over four three-hop paths. The four pairs share a common middle hop with a transmission capacity of R = 300 Mbps. The four links from the servers to the shared link have a transmission capacity of RS = 50 Mbps. Each of the four links from the shared middle link to a client has a transmission capacity of RC = 90 Mbps.

Assuming that the servers are all sending at their maximum rate possible, what are the link utilizations of the shared link (with transmission capacity R)? Enter your answer in a decimal form of 1.00 (if the utilization is 1) or 0.xx (if the utilization is less than 1, rounded to the closest xx).

Your answer: The utilization of shared link is: 0.67

## 10/ COMPUTING UTILIZATION (3).

Consider the scenario shown below, with four different servers connected to four different clients over four three-hop paths. The four pairs share a common middle hop with a transmission capacity of R = 300 Mbps. The four links from the servers to the shared link have a transmission capacity of RS = 50 Mbps. Each of the four links from the shared middle link to a client has a transmission capacity of RC = 90 Mbps.

Assuming that the servers are all sending at their maximum rate possible, what are the link utilizations of the client links (with transmission capacity RC)? Enter your answer in a decimal form of 1.00 (if the utilization is 1) or 0.xx (if the utilization is less than 1, rounded to the closest xx).

Your answer: The utilization of client link is: 0.56

5/ Protocol layers and Their Service Models

## 1/ LAYERS IN THE INTERNET PROTOCOL STACK.

Match the function of a layer in the Internet protocol stack to its its name in the pulldown menu.

Protocols that are part of a distributed network application.: Application Layer

Transfer of data between one process and another process (typically on different hosts).: Transport layer

Delivery of datagrams from a source host to a destination host (typically).: Network layer

Transfer of data between neighboring network devices.: Link layer

Transfer of a bit into and out of a transmission media.: Physical layer

## 2/ WHAT’S A “PACKET” REALLY CALLED?

Match the name of an Internet layer with unit of data that is exchanged among protocol entities at that layer, using the pulldown menu.

Application layer: Message

Transport layer: Segment

Network layer: Datagram

Link layer: Frame

Physical layer: Bit

## 3/ PROTOCOL HEADERS.

Consider the figure below, showing a link-layer frame heading from a host to a router.  There are three header fields shown.  Match the name of a header with a header label shown in the figure.

Header H1: Link layer

Header H2: Network Layer

Header H3: Transport layer

## 4/ WHAT IS "ENCAPSULATION"?

Which of the definitions below describe what is meant by the term "encapsulation"?

Receiving a “packet” from the layer below, extracting the payload field, and after some internal actions possibly delivering that payload to an upper layer protocol.

✅Taking data from the layer above, adding header fields appropriate for this layer, and then placing the data in the payload field of the “packet” for that layer.

Computing the sum of all of the bytes within a packet and placing that value in the packet header field.

Starting a transport layer timer for a transmitted segment, and then if an ACK segment isn’t received before the timeout, placing that segment in a retransmission queue.

Determining the name of the destination host, translating that name to an IP address and then placing that value in a packet header field.

6/ Networks under attack

## 1/ SECURITY DEFENSES.

Match the description of a security defense with its name.

Specialized "middleboxes" filtering or blocking traffic, inspecting packet contents inspections: Firewall

Provides confidentiality by encoding contents: Encryption

Used to detect tampering/changing of message contents, and to identify the originator of a message.: Digital Signatures

Limiting use of resources or capabilities to given users.: Access Control

Proving you are who you say you are.: Authentication

7/History of Computer Networking; Chapter 1 Summary

## 1/ NETWORKING HISTORY - WHEN DID IT HAPPEN?

Match the networking event with the time frame when the event occurred.

Early studies of packet switching by Baran, Davies, Kleinrock.: Early 1960's

First ARPAnet node operational.: Late 1960's

Internetting: DARPA researchers connect three networks together.: 1970's

The Internet Protocol (IP) is standardized in RFC 791.: Early 1980's

Congestion control is added to the TCP protocol.: Late 1980's

The WWW starts up (note: the WWW design started at the end of previous decade).: 1990's

Software-defined networking begins.: 2000-2010

The number wireless Internet-connected devices surpasses the number of connected wired devices.: 2010 - 2020

8 / The Internet Today

## 1/ "WHO CONTROLS THE INTERNET?"

Match an organization name below with the role of the organization in Internet governance. To answer this question you’ll need to watch the Chapter 1 supplemental video on “Who Controls the Internet?”.

1. Internet Governance Forum (IGF)

- A multistakeholder deliberation body, convened by the United Nations, that does not make decisions but informs and inspires those who do.

2. Internet Engineering Task Force (IETF)

- Sets the technical standards for Internet infrastructure -- particularly protocols, device requirements, and data formats -- in more than 9000 Request for Comments (RFCs).

3. **3rd Generation Partnership Project (3GPP)

- Sets the technical standards for 3G, 4G, and 5G mobile cellular systems.

4. Internet Corporation for Assigned Names and Numbers (ICANN)

- Handles (assigns, adjudicates) Internet names, and manages the root level of the DNS.

5. Institute for Electrical and Electronics Engineers (IEEE)

- Sets the technical standard for Ethernet and WiFi link-layer standards.

### CIRCUIT SWITCHING

Consider the circuit-switched network shown in the figure below, with circuit switches A, B, C, and D. Suppose there are 11 circuits between A and B, 13 circuits between B and C, 12 circuits between C and D, and 18 circuits between D and A.

- What is the maximum number of connections that can be ongoing in the network at any one time ?

Ans: The maximum number of connections that can be ongoing at any one time is the sum of all circuits, which happens when 17 connections go from A to B, 14 connections go from B to C, 15 connections go from C to D, and 15 connections go from D to A. This sum is 61.

- Suppose that these maximum number of connections are all ongoing. What happens when another call connection request arrives to the network, will it be accepted? Answer Yes or No

Ans: No, it will be blocked because there are no free circuits.

- Suppose that every connection requires 2 consecutive hops, and calls are connected clockwise. For example, a connection can go from A to C, from B to D, from C to A, and from D to B. With these constraints, what is the is the maximum number of connections that can be ongoing in the network at any one time?

Ans : There can be a maximum of 29 connections. Consider routes A->C and C->A, sum the bottleneck links, consider any leftover capacity that would allow for B->D and D->B connections, and compare that value to the equivalent of B->D and D->B.

- Suppose that 14 connections are needed from A to C, and 17 connections are needed from B to D. Can we route these calls through the four links to accommodate all 31 connections? Answer Yes or No

Ans : Using our answer from question 4, the sum of our needed connections is 31, and we have 29 available connections, so it is NOT possible.

QUANTITATIVE COMPARISON OF PACKET SWITCHING AND CIRCUIT SWITCHING

This question requires a little bit of background in probability (but we'll try to help you though it in the solutions). Consider the two scenarios below:

- A circuit-switching scenario in which Ncs users, each requiring a bandwidth of 25 Mbps, must share a link of capacity 100 Mbps.

- A packet-switching scenario with Nps users sharing a 100 Mbps link, where each user again requires 25 Mbps when transmitting, but only needs to transmit 10 percent of the time.

Round your answer to two decimals after leading zeros

- When circuit switching is used, what is the maximum number of users that can be supported?

Ans : When circuit switching is used, at most 5 users can be supported. This is because each circuit-switched user must be allocated its 20 Mbps bandwidth, and there is 100 Mbps of link capacity that can be allocated.

- Suppose packet switching is used. If there are 9 packet-switching users, can this many user be supported under circuit-switching? Yes or No.

Ans : No. Under circuit switching, the 9 users would each need to be allocated 20 Mbps, for an aggregate of 180 Mbps - more than the 100 Mbps of link capacity available.

- Suppose packet switching is used. What is the probability that a given (specific) user is transmitting, and the remaining users are not transmitting?

Ans : The probability that a given (specific) user is busy transmitting, which we'll denote p, is just the fraction of time it is transmitting, i.e. 0.3. The probability that one specific other user is not busy is (1-p), and so the probability that all of the other Nps-1 users are not transmitting is (1-p)Nps-1. Thus the probability that one specific user is transmitting and the remaining users are not transmitting is p*(1-p)Nps-1, which has the numerical value of 0.017.

- Suppose packet switching is used. What is the probability that one user (any one among the 9 users) is transmitting, and the remaining users are not transmitting?

Ans : Suppose packet switching is used. What is the probability that one user (any one among the 9 users) is transmitting, and the remaining users are not transmitting?

- When one user is transmitting, what fraction of the link capacity will be used by this user? Write your answer as a decimal.

Ans : This user will be transmitting at a rate of 20 Mbps over the 100 Mbps link, using a fraction 0.2 of the link's capacity when busy.

- What is the probability that any 3 users (of the total 9 users) are transmitting and the remaining users are not transmitting?

Ans : The probability that 3 specific users of the total 9 users are transmitting and the other 6 users are idle is p3(1-p)6. Thus the probability that any 4 of the 7 users are busy is choose(9, 3) * p3(1-p)6, where choose(9, 3) is the (9, 3) coefficient of the binomial distribution). The numerical value of this probability is 0.27.

- What is the probability that more than 5 users are transmitting?

Ans : The probability that more than 5 users of the total 9 users are transmitting is Σ i=6,9 choose(9, i) * pi(1-p)9 - i. The numerical value of this probability is 0.025. Note that 5 is the maximum number of users that can be supported using circuit switching. With packet switching, nearly twice as many users (9) are supported with a small probability that more than 5 of these packet-switching users are busy at the same time.

### CAR - CARAVAN ANALOGY

Consider the figure below, adapted from Figure 1.17 in the text, which draws the analogy between store-and-forward link transmission and propagation of bits in packet along a link, and cars in a caravan being serviced at a toll booth and then driving along a road to the next tollbooth.

Suppose the caravan has 20 cars, and that the tollbooth services (that is, transmits) a car at a rate of one car per 2 seconds. Once receiving serving a car proceeds to the next tool both, which is 500 kilometers away at a rate of 20 kilometers per second. Also assume that whenever the first car of the caravan arrives at a tollbooth, it must wait at the entrance to the tollbooth until all of the other cars in its caravan have arrived, and lined up behind it before being serviced at the toll booth. (That is, the entire caravan must be stored at the tollbooth before the first car in the caravan can pay its toll and begin driving towards the next tollbooth).

- Once a car enters service at the tollbooth, how long does it take until it leaves service?

Ans :  Service time is 5 seconds.

- How long does it take for the entire caravan to receive service at the tollbooth (that is the time from when the first car enters service until the last car leaves the tollbooth)?

Ans : It takes 50 seconds to service every car, (10 cars * 5 seconds per car).

- Once the first car leaves the tollbooth, how long does it take until it arrives at the next tollbooth?

Ans : It takes 25 seconds to travel to the next toll booth (500 km / 20 km/s).

- Once the last car leaves the tollbooth, how long does it take until it arrives at the next tollbooth?

Ans :  Just like in the previous question, it takes 25 seconds, regardless of the car.

- Once the first car leaves the tollbooth, how long does it take until it enters service at the next tollbooth?

Ans : It takes 70 seconds until the first car gets serviced at the next toll booth (10-1 cars * 5 seconds per car + 500 km / 20 km/s).

- Are there ever two cars in service at the same time, one at the first toll booth and one at the second toll booth? Answer Yes or No

Ans : No, because cars can't get service at the next tollbooth until all cars have arrived.

- Are there ever zero cars in service at the same time, i.e., the caravan of cars has finished at the first toll both but not yet arrived at the second tollbooth? Answer Yes or No

Ans : Yes, one notable example is when the last car in the caravan is serviced but is still travelling to the next toll booth; all other cars have to wait until it arrives, thus no cars are being serviced.

### COMPUTING THE ONE-HOP TRANSMISSION DELAY

Consider the figure below, in which a single router is transmitting packets, each of length L bits, over a single link with transmission rate R Mbps to another router at the other end of the link.

Suppose that the packet length is L= 4000 bits, and that the link transmission rate along the link to router on the right is R = 1 Mbps.

Round your answer to two decimals after leading zeros

- What is the transmission delay?

Ans : The transmission delay = L/R = 8000 bits / 1000000000 bps = 8.00E-6 seconds .

2. What is the maximum number of packets per second that can be transmitted by this link?

Ans : The number of packets that can be transmitted in a second into the link = R / L = 1000000000 bps / 8000 bits = 125000 packets.

### QUEUING DELAY

Consider the queuing delay in a router buffer, where the packet experiences a delay as it waits to be transmitted onto the link. The length of the queuing delay of a specific packet will depend on the number of earlier-arriving packets that are queued and waiting for transmission onto the link. If the queue is empty and no other packet is currently being transmitted, then our packet’s queuing delay will be zero. On the other hand, if the traffic is heavy and many other packets are also waiting to be transmitted, the queuing delay will be long.

Assume a constant transmission rate of R = 300000 bps, a constant packet-length L = 4600 bits, and a is the average rate of packets/second. Traffic intensity I = La/R, and the queuing delay is calculated as I(L/R)(1 - I) for I < 1.

1. In practice, does the queuing delay tend to vary a lot? Answer with Yes or No

Ans : Yes, in practice, queuing delay can vary significantly. We use the above formulas as a way to give a rough estimate, but in a real-life scenario it is much more complicated. 

2. Assuming that a = 28, what is the queuing delay? Give your answer in milliseconds (ms)

Ans : Queuing Delay = I(L/R)(1 - I) * 1000 = 0.0887*(1900/600000)*(1-0.0887) * 1000 = 0.256 ms.

3. Assuming that a = 52, what is the queuing delay? Give your answer in milliseconds (ms)

Ans : Queuing Delay = I(L/R)(1 - I) * 1000 = 0.1647*(1900/600000)*(1-0.1647) * 1000 = 0.4357 ms. 


4. Assuming the router's buffer is infinite, the queuing delay is 0.4357 ms, and 1218 packets arrive. How many packets will be in the buffer 1 second later?

Ans : Packets left in buffer = a - floor(1000/delay) = 1218 - floor(1000/0.4357) = 0 packets

5. If the buffer has a maximum size of 563 packets, how many of the 1218 packets would be dropped upon arrival from the previous question?

Ans : Packets dropped = packets - buffer size = 1218 - 563 = 655 dropped packets.

### COMPUTING END-END DELAY (TRANSMISSION AND PROPAGATION DELAY)

Consider the figure below, with three links, each with the specified transmission rate and link length.

Assume the length of a packet is 8000 bits. The speed of light propagation delay on each link is 3x10^8 m/sec

Round your answer to two decimals after leading zeros

- What is the transmission delay of link 1?

Ans : Link 1 transmission delay = L/R = 8000 bits / 10 Mbps = 0.0008 seconds

- What is the propogation delay of link 1?

Ans : Link 1 propagation delay = d/s = ()3 Km) * 1000 / 3*10^8 m/sec = 1.00E-5 seconds

3. What is the total delay of link 1?

Ans : Link 1 total delay = d_t + d_p = 0.0008 seconds + 1.00E-5 seconds = 0.00081 seconds

4. What is the transmission delay of link 2?

Ans : Link 2 transmission delay = L/R = 8000 bits / 100 Mbps = 8.00E-5 seconds 

5. What is the propogation delay of link 2?

Ans : Link 2 propagation delay = d/s = ()500 Km) * 1000 / 3*10^8 m/sec = 0.0017 seconds

6. What is the total delay of link 2?

Ans : Link 2 total delay = d_t + d_p = 8.00E-5 seconds + 0.0017 seconds = 0.0017 seconds

7. What is the transmission delay of link 3?

Ans : Link 3 transmission delay = L/R = 8000 bits / 1000 Mbps = 8.00E-6 seconds

8. What is the propogation delay of link 3?

Ans : Link 3 propagation delay = d/s = ()3 Km) * 1000 / 3*10^8 m/sec = 1.00E-5 seconds

9. What is the total delay of link 3?

Ans : Link 3 total delay = d_t + d_p = 8.00E-6 seconds + 1.00E-5 seconds = 1.80E-5 seconds

10. What is the total delay?

Ans : The total delay = d_L1 + d_L2 + d_L3 = 0.00081 seconds + 0.0017 seconds + 1.80E-5 seconds = 0.0026 second.

### END TO END THROUGHPUT AND BOTTLENECK LINKS

Consider the scenario shown below, with four different servers connected to four different clients over four three-hop paths. The four pairs share a common middle hop with a transmission capacity of R = 400 Mbps. The four links from the servers to the shared link have a transmission capacity of RS = 80 Mbps. Each of the four links from the shared middle link to a client has a transmission capacity of RC = 30 Mbps.

You might want to review Figure 1.20 in the text before answering the following questions

1. What is the maximum achievable end-end throughput (in Mbps) for each of four client-to-server pairs, assuming that the middle link is fairly shared (divides its transmission rate equally)?
Ans: The maximum achievable end-end throughput is the capacity of the link with the minimum capacity, which is 20 Mbps.

2. Which link is the bottleneck link? Format as Rc, Rs, or R
Ans: The bottleneck link is the link with the smallest capacity between RS, RC, and R/4. The bottleneck link is Rc.

3. Assuming that the servers are sending at the maximum rate possible, what are the link utilizations for the server links (RS)? Answer as a decimal
Ans: The server's utilization = Rbottleneck / RS = 20 / 50 = 0.4

4. Assuming that the servers are sending at the maximum rate possible, what are the link utilizations for the client links (RC)? Answer as a decimal
Ans: The client's utilization = Rbottleneck / RC = 20 / 20 = 1

5. Assuming that the servers are sending at the maximum rate possible, what is the link utilizations for the shared link (R)? Answer as a decimal

Ans: The shared link's utilization = Rbottleneck / (R / 4) = 20 / (300 / 4) = 0.27

THE IP STACK AND PROTOCOL LAYERING

In the scenario below, imagine that you're sending an http request to another machine somewhere on the network.

1. What layer in the IP stack best corresponds to the phrase: 'bits live on the wire'
Ans: The given phrase corresponds to the Physical Layer.
 
2. What layer in the IP stack best corresponds to the phrase: 'passes frames from one node to another across some medium'
Ans: The given phrase corresponds to the Link Layer.

3. What layer in the IP stack best corresponds to the phrase: 'moves datagrams from the source host to the destination host'
Ans: The given phrase corresponds to the Network Layer.

4. What layer in the IP stack best corresponds to the phrase: 'handles messages from a variety of network applications'
Ans: The given phrase corresponds to the Application Layer.

5. What layer in the IP stack best corresponds to the phrase: 'handles the delivery of segments from the application layer, may be reliable or unreliable'
Ans: The given phrase corresponds to the Transport Layer.

6. What layer corresponds to box 1?
Ans: Box 1 is the Application Layer.

7. What layer corresponds to box 2?
Ans: Box 2 is the Transport Layer.

8. What layer corresponds to box 3?
Ans: Box 3 is the Network Layer.

9. What layer corresponds to box 4?
Ans: Box 4 is the Link Layer.

10. What layer corresponds to box 5?
Ans: Box 5 is the Physical Layer.

11. What layer corresponds to box 6?
Ans: Box 6 is the Physical Layer.

12. What layer corresponds to box 7?
Ans: Box 7 is the Link Layer.

13. What layer corresponds to box 8?
Ans: Box 8 is the Physical Layer.

14. What layer corresponds to box 9?
Ans: Box 9 is the Link Layer.

15. What layer corresponds to box 10?
Ans: Box 10 is the Network Layer.

16. What layer corresponds to box 11?

Ans: Box 11 is the Physical Layer.


17. What layer corresponds to box 12?
Ans: Box 12 is the Link Layer.

18. What layer corresponds to box 13?
Ans: Box 13 is the Network Layer.

19. What layer corresponds to box 14?
Ans: Box 14 is the Transport Layer.

20. What layer corresponds to box 15?
Ans: Box 15 is the Application Layer.